Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FIREPLACE SURROUNDS AND HEARTHS

- ESTABLISH A LINE WHERE THE WALL IS TO BE LOCATED. PLACE THE FIREBOX FLUSH WITH THE FINISHED WALL

- PLACE THE METAL FLASHING HALFWAY BENEATH THE FIREBOX. PLACE THE HEARTH EXTENSION OVER THE METAL FLASHING AND SECURE TO THE FLOOR WITH ADHESIVE OR BY SECURING CARPET AROUND THE OUTER EDGE OF THE HEARTH EXTENSION.

- PLACE THE SURROUND ON TOP OF THE HEARTH CENTERING ON THE FIREBOX AND SECURE TO THE STUD WALL WITH (5) SCREWS THRU GROUT LINE.

- PLACE THE MANTEL ON TOP OF THE SURROUND AND SECURE TO THE STUD WALL WITH (2) SCREWS.

- IT IS RECOMMENDED THAT GROUT BE USED TO COVER SCREWS. THIS GROUT IS AVAILABLE THROUGH CLASSIC SURROUNDS.

- REFER TO THE FIREBOX INSTALLATION MANUAL FOR FURTHER REQUIREMENTS FROM THE FIREBOX MANUFACTURER.