Classic Surrounds LLC

INSTRUCTIONS

FOR CUTTING INTERIOR AND EXTERIOR PANELS

RECOMMENDED TOOL LIST

- SKILL SAW

- WOOD SKILL SAW BLADE

- DRY CUT MASONARY BLADE

- MEASURING TAPE

- CHALK LINE

- SAFETY GLASSES

- PLACE PANEL WITH BACKSIDE UP, STONE SIDE DOWN. ALL CUTS SHOULD BE PUT ON A SOFT CUTTING SURFSCE SUCH AS CARPET, STYROFOAM, ETC.(CUT PIECES NEED TO BE SUPPORTED WHILE CUT IS BEING MADE)

- MARK BACK PANEL TO BE CUT WITH CHAULK LINE.

- SET WOOD SKILL SAW BLADE TO 5/8 TO 11/16 MAX. (SEE FIG.1)

- MAKE CUT THROUGH WOOD.

- CHANGE WOOD BLADE TO MASONARY BLADE.

- SET MASONARY BLADE TO FULL CUT

- FOLLOW SAME CUT AS WOOD BLADE CUTTING THRU STONE.

PAGE 10