Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR MANTELS

- PLACE MANTEL OVER SURROUND BEING SURE THE BOTTOM PART OF THE BOX ON THE BACK OF THE MANTEL RESTS UPON THE TOP OF THE SURROUND

- PENCIL MARK ON THE WALL WHERE THE TOP SHELF AND WALL MEETS.

- REMOVE MANTEL AND MEASURE DOWN ¾ OF AN  INCH OR THE THICKNESS OF THE TOP SHELF

- USING A 2 X 2 PIECE OF DEADWOOD CUT APPROXIMATELY THE LENGTH OF THE INSIDE OPENING OF THE BOX. ATTACH TO THE WALL BY SCREWING IT INTO THE STUDS ¾ OF AN INCH BELOW THE PENCIL MARKS.

- PLACE MANTEL BACK ON TOP OF THE SURROUND SO THAT THE BOTTOM OF THE TOP SHELF RESTS UPON THE PIECE OF DEADWOOD.

- CENTER MANTEL AND ATTACH BY SCREWING THE MANTEL (THROUGH THE BUTTON HOLES) INTO THE DEADWOOD.

- COVER THE HOLES WITH THE BUTTONS PROVIDED.

PAGE 11