Classic Surrounds LLC

REPAIR INSTRUCTIONS

FOR BRICK

- WITH MASONRY BLADE CUT MORTAR JOINT AROUND DEFECTIVE BRICK TO PREVENT SURROUNDING BRICK FROM CRACKING.

- USING A HAMMER AND CHISEL, KNOCK BRICK OUT MAKING SURE SURFACE UNDERNEATH IS LEVEL.

- APPLY LIQUID NAIL TO SURFACE AND TO BACK OF BRICK AND PLACE BRICK TO SURFACE.

- ALLOW LIQUID NAIL TO OOZE OUT OT INSURE PROPER BONDING.

- COMBINE DRY MIX WITH WATER TO DESIRED PASTE AND APPLY TO JOINTS AROUND BRICKS.

- WITH S-JOINTER SMOOTH GROUT.

- CLEAN BRICK

PAGE 12