Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR BRICK SURROUNDS

- PLACE HEARTH ON FLOOR APPROXIMATELY 1” FROM WALL.

- SET SURROUND DIRECTLY ON TOP OF HEARTH MAKING SURE 1” METAL FLANGE AT BOTTOM OF LEG LOCKS BEHIND HEARTH.

- PUSH HEARTH TIGHTLY AGAINST WALL. INSTALL (2) #8x1-1/2” SCREWS IN METAL GLANGE THAT FOLDS FROM UNDERNEATH THE HEARTH.

- PRESS TOP OF SURROUND AGAINST WALL. APPLY (2) SCREWS THAT ARE 3” OR LONGER AT A 45 °ANGLE THROUGH 2X4 INTO WALL.

- SET MANTEL ON TOP OF 2X4 MAKING SURE IT IS CENTERED AND INSERT (2) 1-1/2” SCREWS INTO PREDRILLED HOLES.

- COVER SCREWS WITH BUTTONS.

PAGE 14