Classic Surrounds LLC

INSTRUCTIONS

STONE REPAIR KITS

- TO REPAIR CHIPPED STONES

  - MIX STONE DYE THAT IS IN THE WHITE ENVELOPE WITH WATER

  - MAKE SURE COLOR IS RIGHT. ADD WATER AS NEEDED TO GET THE DESIRED COLOR.

  - USE A PAINT BRUSH TO PUT ON CHIPPED STONE AND WIPE OFF WITH SPONGE.

- TO REPAIR FALLEN STONES

  - CLEAN AREA WHERE STONE HAS FALLEN

  - APPLY TUBE GROUT ON THE BACK SO THE PROPER SIZED STONE

  - PLACE STONE IN AREA. THEN CLEAN EXCESS GROUT WITH SPONGE.

- TO REPAIR GROUT AREA

  - MIX GROUT MIXTURE WITH THE LATEX THAT IS SUPPLIED BY CLASSIC SURROUNDS.

  - MAKE SURE COLOR IS MATCHED. ADD LATEX AS NEEDED TO ADJUST COLOR AND TEXTURE.

  - APPLY TO AREA WHERE GROUT IS NEEDED.

PAGE 16