Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FIREPLACE SURROUNDS AND HEARTHS

# WOOD / TILE PRODUCTS

- ESTABLISH A LINE WHERE THE WALL IS TO BE LOCATED. PLACE THE FIREBOX FLUSH WITH THE FINISHED WALL. (IF USING A RAISED HEARTH, BUILD A 6” PLATFORM TO PLACE THE FIREBOX ON.)

- BUILD A FRAME WALL AROUND THE FIREBOX, USING A MINIMUM OF 2” TO 3” STUDS, POSITIONING THE STUDS IN PLACE SO YOU CAN SCREW THROUGH THE OUTER EDGE OF THE TILE AND WOOD SURROUND INTO THE FRAME WALL.

- PLACE THE METAL FLASHING (INCLUDED ON THE RAISED HEARTH) HALFWAY BENEATH THE FIREBOX. PLACE THE HEARTH EXTENSION OVER THE METAL FLASHING AND SECURE TO THE FLOOR WITH A NON-FLAMMABLE ADHESIVE, A DOUBLE-SIDED TAPE OR BY SECURING CARPET AROUND THE OUTER EDGES OF THE HEARTH EXTENSION.

- PLACE THE TILE SURROUND ON TOP OF THE HEARTH SURROUNDING THE FIREBOX AND SECURE TO THE WALL WITH (6) SCREWS. THE SCREWS SHOULD BE PLACED ALONG THE OUTER EDGE OF THE SURROUND WHERE THERE IS NO TILE INTERFERENCE. (2) SCREWS SHIOULD BE USED ON TOP AND (2) SCREWS ON EACH SIDE. (OPTIONAL) DRILL A PILOT HOLE IN CENTER OF GROUTLINE USING

(CONTUIED)

PAGE 17

A 1/16” DRILL BIT. USE A FLATHEAD SCREW TO MOUNT SURROUND TO THE WALL THROUGH PILOT HOLE. THIS WILL PREVENT TILE CRACKING.

- THE WOOD SURROUND SHOULD BE ASSEMBLED TOGETHER AND THEN LAID AGAINST THE WALL AROUND THE TILE SURROUND. SECURE THE WOOD SURROUND TO THE FRAME WALL PREVIOUSLY POSITIONED WITH (6) SCREWS.

- IT IS RECOMMENDED THAT GROUT BE USED TO FILL IN BETWEEN THE FIREBOX AND HEARTH AND OVER ALL SCREWS. THIS GROUT SHOULD BE A NON-FLAMMABLE TYPE, AVAILABLE THROUGH CLASSIC SURROUNDS. THE GROUT SHOULD NOT BE PLACED ABOVE THE LOUVERS WHERE IT MAY CAUSE INTERFERENCE. REFER TO THE FIREBOX INSTALLATION MANUAL FOR FURTHER REQUIREMENTS FROM THE FIREBOX MANUFACTURER.

PAGE 18