Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

STONE MANTEL WITH BRACKETS

- MOUNT BRACKETS TO STONE SURROUND

  - LAY SURROUND ON A FLAT TABLE ALLOWING ACCESS TO PREDRILLED HOLES ON SURROUND.

  - PLACE BRACKET ON SURROUND IN PREDRILLED HOLES TO ASSURE CORRECT ALIGNMENT WITH TOP EDGE OF SURROUND.

  - ONCE ALIGNMENT IS CHECKED REMOVE BRACKET AND APPLY MATCHING GROUT TO METAL PLATE. APPLYING GROUT AROUND OUTSIDE EDGES AND ONE BEAD IN THE MIDDLE OF PLATE.

  - PLACE BRACKET ON SURROUND THRU PREDRILLED HOLES. MAKE SURE ALIGNMENT IS CORRECT AND GROUT IS “OOZING” FROM EDGES. A SMALL SQUARE SHOULD BE USED TO INSURE TOP OF BRACKET IS SQUARE WITH BACK OF SURROUND. YOU MADE NEED TO USE A WASHER ON THE TOP OR BOTTOM OF BRACKET IF IT IS NOT SQUARE.

  - PLACE WASHERS AND NUTS ON BOLTS THAT EXTEND THRU THE BACK OU SURROUND AND TIGHTEN SECURELY.

- INSTALL SURROUND

  - ATTACH SURROUND TO STUD WALL USING STANDARD PROCEDURE. LEAVE SCREWS AT TOP OF SURROUND LOOSE.

(CONTUIED)

PAGE 21

  - SO METAL FLANGE ON MANTEL WILL SLIDE BEHIND SURROUND.

- MOUNT MANTEL TO SURROUND AND BRACKETS

  - PLACE A BEAD OF **GROUT ON THE FULL LENGTH OF TOP OF SURROUND.

  - PLACE A BEAD OF **GROUT DOWN CENTER AND OUTSIDE EDGES OF BRACKETS.

  - PLACE STONE MANTEL ON SURROUND WITH SOMEONE ON EACH END MAKING SOURTHE MANTEL IS PLACED STRAIGHT DOWN ON THE TOP OF SURROUND. ALLOWING METAL FLANGE TO GO BEHIND SURROUND. TIGHTEN TOP SCREWS.

NOTE: GROUT WILL BE CURED 25% IN APPOXIMATELY 2HRS AND

100% IN 24HRS.

** USE “COLOR GROUT” OR EQUIVALENT ONLY

PAGE 22