Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR STONE BOOTS

- AFTER HEARTH HAS BEEN INSTALLED, CENTER SURROUND IN THE DESIRED LOCATION

- PENCIL MARK APPROXIMATELY ONE INCH ABOVE THE HEARTH ON EACH SIDE OF THE LEG SO YOU WILL KNOW WHERE TO PLACE THE BOOT ONCE SURROUND HAS BEEN MOVED.

- REMOVE SURROUND FROM THE WALL

- APPLY ADHESIVE (LIQUID NAIL) TO THE BACK OF THE BOOT AND PLACE BETWEEN THE PENCIL MARKS ON THE WALL.

- SECURE BOOT TO THE WALL WITH A COUPLE OF SCREWS AND COVER SCREW HOLES WITH CAULK AND ANY GAPS OR CRACKS.

- ONCE BOTH BOOTS ARE IN PLACE, CAREFULLY PLACE SURROUND ON TOP OF THE BOOTS BEING SURE SURROUND AND BOOTS LINE UP.

- SECURE SURROUND TO THE WALL.

PAGE 23