Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR RAISED PANEL SURROUND

- PLACE HEARTH ON FLOOR CENTERED WITH FIREBOX.

- PLACE SIDE PIECES OF METAL ON THE BACK INSIDE LEGS OF SURROUND.

- ALLOW METAL TO OVERLAP ONSIDE LEGS BY ONE INCH.

- STAPLE METAL TO THE BACK OF THE SURROUND

- STAPLE TOP PIECE OF METAL ON BACK OF SURROUND TO OVERHANG FIREBOX 1/2 “

- PLACE SURROUND ON TOP OF HEARTH MAKING SURE IT IS CENTERED

- SECURE SURROUND TO WALL WITH SCREWS IN PREDRILLED HOLES IN THE LEGS.

- PLACE (2) 3” SCREWS IN MANTEL BRACKETS AND SECURE TO WALL.

- COVER SCREWS HOLES WITH BUTTONS.

PAGE 25