Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR ARLINGTON SURROUND

- PLACE HEARTH ON FLOOR CENTERED WITH FIREBOX.

- ASSEMBLE SURROUND BY ATTACHING THE LEGS TO THE BREASTPLATE

- DRILL #8 x 1-1/4 SCREWS INTO BACK OF BREASTPLATE TO SECURE LEGS.

- STAPLE TOP PIECE OF METAL ON WALL ACROSS THE TOP OF THE FIREBOX TO OVERHANG FIREBOX ¾”.

- STAPLE METAL IN PLACE TO THE WALL.

- PLACE THE WOOD SURROUND ON TOP OF THE HEARTH AND CENTER WITH THE FIREBOX.

- PREDRILL HOLES AT STUD LOCATIONS AND SCREW SURROUND TO WALL WITH 2-1/2” WOOD SCREWS.

- COVER SCREWS HOLES WITH BUTTONS.

PAGE 28