Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR ACORN SURROUND

- ONCE HEARTH IS IN PLACE ATTACH BLACK METAL TRIM KIT AROUND OUTER EDGE OF THE FIREPLACE USING PIN NAILS.

- THE ACORN CONSISTS OF (3) PIECES, THE BREASTPLATE AND TWO LEGS. ALIGN THE LEFT AND RIGHT LEG WITH THE BREASTPLATE AND ATTACH USING 1-1/4” SCREWS OR STAPLES.

- BUTTON HOLES ARE NOT PREDIRLLED. CENTER SURROUND AND PRE-DRILL HOLES ACCORDING TO STUDS IN THE WALL. ATTACH SURROUND TO THE WALL WITH SCREWS AND COVER HOLES WITH BUTTONS PROVIDED. TWO SCREWS IN BREASTPLATE AND ONE IN EACH LEG NEAR THE BOTTOM.

PAGE 30