Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR PLANTATION SURROUND

- PLACE HEARTH ON FLOOR CENTERED WITH FIREBOX.

- PLACE SIDE PIECES OF METAL ON THE BACK INSIDE LIGS OF SURROUND

- ALLOW METAL TO OVERHAND INSIDE LEGS BY ONE INCH

- STAPLE METAL TO THE BACK OF SURROUND

- STAPLE TOP PIECE OF METAL ON BACK OF SURROUND TO OVERHANG FIREBOX ½”.

- PLACE SURROUND ON TOP OF HEATH MAKING SURE IT IS CENTERED.

- SECURE SURROUND TO WALL WITH SCREWS IN PREDRILLED HOLES IN THE LEGS.

- PLACE (2) #8 x 3” SCREWS UNDERNEATH TOP SHELF AND SECURE TO WALL.

- COVER SCREW HOLES UNDERNEATH TOP SHELF WITH CROWN MOLDING USING ONE-INCH BRADS.

PAGE 31