Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FOR SKIRTED CROWN MANTELS

- INSTALL HEARTH EXTENSIONS AND STONE SURROUND.

- SET MANTEL ON TOP OF SURROUND ALLOWING SKIRT TO CAP OVER SURROUND. (SEE ILLUSTRATION)

- MARK WALL OVER BUTTON HOLES WITH PENCIL.

- TAKE MANTEL DOWN

- MEASURE DOWN ¾” FROM MARKS.

- SECURE DEAD WOOD TO WALL WITH #8 x 3” WOOD SCREWS (MIN 2 PER STUD).

- PLACE MANTEL ON SURROUND ALLOWING TOP SHELF TO SET ON TOP OF DEAD WOOD.

- ATTACH MANTEL TO DEAD WOOD WITH #8 x 1-1/2” FLAT HEAD WOOOD SCREWS.

- FILL SCREW HOLES WITH PROVIDED BUTTONS.

PAGE 32