Classic Surrounds LLC

INSTALLATION INSTRUCTIONS

FIREPLACE SURROUNDS AND HEARTHS

# WOOD  PRODUCTS

- ESTABLISH A LINE WHERE THE WALL IS TO BE LOCATED. PLACE THE FIREBOX FLUSH WITH THE FINISHED WALL. (IF USING A RAISED HEARTH, BUILD A 6” PLATFORM TO PLACE THE FIREBOX ON.)

- BUILD A FRAME WALL AROUND THE FIREBOX, USING A MINIMUM OF 2” TO 3” STUDS,

- PLACE THE METAL FLASHING (INCLUDED ON THE RAISED HEARTH) HALFWAY BENEATH THE FIREBOX. PLACE THE HEARTH EXTENSION OVER THE METAL FLASHING AND SECURE TO THE FLOOR WITH A NON-FLAMMABLE ADHESIVE, A DOUBLE-SIDED TAPE OR BY SECURING CARPET AROUND THE OUTER EDGES OF THE HEARTH EXTENSION.

- PLACE THE SURROUND ON TOP OF THE HEARTH SURROUNDING THE FIREBOX AND SECURE TO THE WALL WITH (4) SCREWS. THE SCREWS SHOULD BE PLACED IN THE PREDRILLED HOLES.

- COVER SCREW HEADS WITH PROVIDED BUTTONS.